Isaias Osorio

CPE301 – SPRING 2016

# Design Assignment 4

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST |  |  |
| 1. | INITIAL CODE OF TASK 1 |  |  |
| 2. | SCHEMATICS |  |  |
| 3. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 4. | SCREENSHOT OF EACH DEMO |  |  |
| 5. | VIDEO LINKS OF EACH DEMO |  |  |
| 6. | GITHUB LINK OF THE DA |  |  |

| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| --- | --- | --- | --- |

COMPONENTS:

ATmega328P Chip x1

RGB LED x1

100 Ω resistor x2

150 Ω resistor x1

| 1. | INITIAL CODE OF TASK 1/A |  |  |
| --- | --- | --- | --- |

\* DA-4.c

* Created: 4/10/2016 3:11:05 PM

* Author : Isaias Osorio

*/

#define F_CPU 8000000UL // frequency for delay functions

#include <util/delay.h> // library for _delay_ms() function

#include <avr/io.h> // library for I/O

void delay (); // function declaration for delay

int main(void)

{

unsigned char i = 230, j=230, k=230;// initialize to 230 for 10% value of 255

unsigned char counter2 = 0, counter3 = 0;

// two variables to update 2nd and 3rd PWMs counters

DDRD = 0x40;		// OC0A

DDRB = 0x0A;		// OC1A and OC2A

OCR0A = 230;		// 230 is 10% value of 255 when decrementing

TCCR0A = 0xC1;	// Phase Correct PWM, inverted

TCCR0B = 0x03;	// N = 64

OCR1A = 230;		// 230 is 10% value of 255 when decrementing

TCCR1A = 0xC1;	// Phase Correct PWM, inverted

TCCR1B = 0x03;	// N = 64

OCR2A = 230;		// 230 is 10% value of 255 when decrementing

TCCR2A = 0xC1;		// Phase Correct PWM, inverted

TCCR2B = 0x04;		// N = 64

while (1) // loop forever

{

OCR0A = i;		// update 3 PWM values at beginning of each loop

OCR1A = j;		// from delay of 40ms

OCR2A = k;

counter2++;	// increment counter for 2nd PWM every time 1st is updated

i -= 5;		// increase duty cycle of PWM1 by 10%

if(i < 30)		// if duty cycle is more than 90% (inverted)

i = 230;	// reset duty cycle back to 10%

if(counter2 >= 9)

{				// if PWM1 reset duty cycle

counter2 = 0; // clear counter for PWM2

counter3++; 	// increment counter for PWM3

j -= 5;	// increase duty cycle of PWM2 by 10%

if(j < 30)	// if duty cycle is more than 90% (inverted)

j = 230; // reset duty cycle back to 10%

}

if(counter3 >= 9)

{			// if PWM2 reset duty cycle

counter3 = 0;	// clear counter for PWM3

k-=5;		// increase duty cycle of PWM3 by 10%

if(k < 30)	// if duty cycle is more than 90% (inverted)

k = 230;// reset duty cycle back to 10%

}

_delay_ms(40);	// delay for 40ms

}

return 0;

}

| 2. | SCHEMATICS |  |  |
| --- | --- | --- | --- |

| 3. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

TASK 1: Showing the value i being output to OCR0A as 30 which will be 90% duty cycle using inverted mode.

| 4. | SCREENSHOT OF EACH DEMO |  |  |
| --- | --- | --- | --- |

BREADBOARD: Actual circuit is the ATmega328P connected to RGB with 3 resistors. Image was taken during mid sequence of the RGB.

| 5. | VIDEO LINKS OF EACH DEMO |  |  |
| --- | --- | --- | --- |
| https://github.com/isaiasmoioso/CPE-301/DA-4/IMG_3748.MOV |  |  |  |
| 6. | GITHUB LINK OF THE DA |  |  |
| https://github.com/isaiasmoioso/CPE-301.git |  |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Isaias Osorio